DIVISION 32 30 00 SITE IMPROVEMENTS

SECTION 32 33 00 SITE FURNISHINGS

SECTION 32 33 43 SITE SEATING AND TABLES

SECTION 32 33 43.13 SITE TABLES

SECTION-1 – GENERAL

1.1 SUMMARY

A. This section includes the following:

1. Tables (Model 295 series)

1.2 QUALITY ASSURANCE

A. Installer Qualification: An experienced installer who has completed installation of site furnishings and whose

work has resulted in construction with a record of successful in-service performance.

B. Manufacturer Qualifications: Experienced site furniture manufacturer since 1984.

1.3 SUBMITTALS

A. Product Data: Include physical characteristics such as shape, dimensions and finish for each bench.

B. Shop Drawings: Provide installation details for each product.

C. Samples for Verification: For the following product, show the color of the powder coat finish.

D. Maintenance Data: For each product.

1. Provide recommended methods for repairing damage and abrasions to the powder coat finish.

1.4 DELIVERY, STORAGE, AND HANDLING

A. Store products in original undamaged packaging in a dry location until ready for installation.

B. Handle powder coated products with carefully to prevent any damage to the finish.

1.5 WARRANTY

A. All products manufactured by DuMor, Inc., are warrantied against defect in materials and/or workmanship and in

accordance with our published specifications. DuMor, Inc. further warrants our products as follows:

1. Limited twenty-year warranty against structural failure of all steel bench frames or complete steel

bench assemblies, table frames, litter receptacle frames, steel planters and all cast iron and aluminum

bench supports.

2. Limited five-year warranty against structural failure of wood slats.

3. Limited ten-year warranty against structural failure of recycled plastic. It is further warrantied not to

degrade, split, crack or splinter during this period.

4. Limited three-year warranty on structural failure of all bike racks.

5. Limited one-year warranty on any item not specifically discussed above.

SECTION-2 - PRODUCTS

2.1 ACCEPTABLE MANUFACTURERS

A. Provide products from the following manufacturer:

1. DuMor Inc.

138 Industrial Circle

Mifflintown, PA 17059

Phone: 800-598-4018

Fax: 717-436-9839

Email: sales@dumor.com

Website: www.dumor.com

2.2 TABLES - DuMor Model 295 Series

A.  Materials:

1. Supports:

a. Table supports shall be manufactured from 2” (2 3/8” OD) ASTM A513 schedule 40 steel tubing and 3/8”

thick ASTM A36 steel plate.

2. Seat Assembly Wood Slat:

a. Seat surface shall be manufactured from 1” x 2 1/2” (finished) wood slats and 3/16” thick ASTM A36 steel

plate.

b. Seat frame shall be manufactured from 1/4” thick ASTM A36 steel plate and 3/8” thick ASTM A36 steel

plate.

3. Seat Assembly Horizontal Strap “HS”:

a. Seat surface shall be manufactured from 3/16” thick ASTM A36 steel plate.

b. Seat frame shall be manufactured from 1/4” thick ASTM A36 steel plate and 3/8” thick ASTM A36 steel

plate.

4. Table Top:

a. Table top shall be manufactured from 10 gauge ASTM A1011 formed steel plate.

5. Anchoring:

a. Stainless steel expansion anchors (1/2” x 3 3/4”) provided.

b. Nylon glides (1/2” x 2 1/2”) provided.

B. Dimensions

1. Overall:

a. 6 seat table: 81 9/16” long x 81 9/16” deep x 30” high.

b. 5 seat table: 82 9/16” long x 82 9/16” deep x 30” high.

c. 4 seat table: 81 9/16” long x 81 9/16” deep x 30” high.

d. 3 seat (ADA compliant) table: 81 9/16” long x 81 9/16” deep x 30” high.

C. Finish:

1. Powder Coating

a. All parts are processed through an 8-stage iron phosphorous wash system.

b. Parts are coated with a zinc-rich epoxy primer to an AVERAGE of 4-5 mils.

c. Parts are then finished with a top coat of TGIC-polyester powder to an AVERAGE of 4-5 mils.

d. Powder is cured at the powder manufacturers specifications using combination of infrared and convection

heat for approximately 20 minutes.

e. Finished parts shall comply with the following American Standard Test Method (ASTM) for coating and

coating method:  ASTM-D-523, ASTM-D-3363, ASTM-D-1737, ASTM-D-3359, ASTM-D-2794, ASTM-

B-117 and ASTM-D-3451.

SECTION-3 – EXECUTION

3.1 INSTALLATION

A. Handle and install products according to manufacturer’s recommendations and installation instructions.

B. Some assembly required.